SECTION 32 31 13

FENCE AND GATES

PART 1 – GENERAL

  - DESCRIPTION

- This section describes the following fence system:

- SecuriMesh D-Fence D-Lay 5+ M Bent fencing panels fabricated with welded galvanized wire mesh including galvanized fence posts. SecuriMesh D-Fence D-Lay 5+ M bent fence shall be furnished and installed as shown on the plans and specified herein, overall height of the fence shall be 4ft to 15ft tall.

  - REQUIREMENTS

- Furnish materials, labor, expertise and equipment necessary to complete all work 	specified in this section and as shown on the drawings.

- Structural Performance: Provide product and installation capable of withstanding the 	effects of gravity lads and the following loads and stresses within limits and under conditions indicated.

    - Uniform pressure of 30 lbf/sq. ft. acting inward or outward.

    - Thermal Movements resulting from a temperature change (range) of 120 degrees Fahrenheit ambient and 180 degrees Fahrenheit material surfaces.

  - SUBMITTALS

- Shop drawings and manufacturer's literature: Provide specifications and construction 	detail drawings to substantiate quality of materials and provide details of fabrication and 	installation.

- Submittals shall be in accordance with standard construction practices to include 	complete detailed layout of all panels & posts.  Submittals shall include plan layout, 	elevations and section views of panels & posts.

- Certificate: manufacturer’s certification that materials meet specification requirements.

  - REFERENCES

- ANSI B36.10 & 36.19 – Standard Specification for the dimensions of the steel pipe.

- ASTM A123/A123M – Standard Specification for Steel, Zinc-Coated (Galvanized) by 	Hot-Dip Process

- ASTM D7091 –Standard Practice for Nondestructive Measurement of Dry Film 	Thickness of Nonmagnetic Coatings Applied to Ferrous Metals and Nonmagnetic, 	Nonconductive Coatings Applied to Non-Ferrous Metals.

- ASTM D1729 –Standard Practice for Visual Appraisal of Colors and Color Differences 	of Diffusely-Illuminated Opaque Materials.

  - QUALITY ASSURANCE

- Installation of fence and materials shall conform to the requirements of the fence 	manufacturer.

- The fence shall be warranted from any defects in materials and workmanship for a period 	as specified in the relevant section of the contract documents.

PART 2 – PRODUCTS

  - MANUFACTURERS

- Security bent fence:

Acceptable Manufacturers:

Bear Mountain Security, LLC

7400/7404 Gary Ave, Miami Beach, FL 33141

202-549-3431

www.BearMountainSecurity.com

2.2 COMPONENTS

  - Panel:

    - Panel Height: 4ft to 15ft

    - Panel Width: 10ft.

    - Panel shall be of welded wire mesh of size 3 inch x 0.5 inch with the wire of dia 	11ga to 	4ga.

  - Fence Posts:

    - Post shall be available in two types:

1. 	Circular Hollow Section, Posts shall be based on applicable height & wind load, CR20 & 	CR40 	of SCH40 & SCH80 pipe in carbon steel.

2.	 C Channel, Posts shall be 3.2 inch x 2.5 inch, C Channel in carbon steel.

    - The post shall be galvanized after fabrication. Top covered with pvc cap. Length as 	specified on the contract drawings.

    - Post center spacing shall be 9.675ft.

  - Fittings and accessories:

    - The clamps & cover plates shall be galvanized and sized as specified by the fence 	manufacturer.

    - For Circular Hollow Section Post, fence panels shall be attached to posts using the clamp, 	which will be secured with 5/16-inch x 2 ⅜ inch stainless steel mushroom head bolts, 	accompanied by security nuts and washers.

    - For C Channel Post, fence panels shall be attached to posts using the cover plate, which 	will be secured with 5/16-inch x 4 ¾ inch stainless steel mushroom head bolts, 	accompanied by security nuts and washers.

PART 3 – EXECUTION

  - INSPECTION

  - Verify that final grading in fence location is completed and without irregularities which 	will interfere with fence installation. Security bent fence is designed to be installed on a 	level surface. Variations in height, slopes, stairs, and steeping shall be shown on contract 	drawings and on submittal drawings.

  - Field verify all fence dimensions and layout prior to commencing installation.

  - Do not commence work until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

- Install the fence following the manufacturer’s installation instructions; for detailed		guidance, refer to the installation manual.

- Install fence plumb and level. Posts are to be embedded inside the ground.

- Do not install bent, bowed or otherwise damaged panels. Remove damaged	         	components from site and replace.

- For Circular Hollow Section Post, secure fence panels to the fence posts using 		stainless steel 5/16-inch x 2 ⅜ inch mushroom head bolts, accompanied by			security nuts and washers, with the help of a clamp.

- For C Channel Post, secure fence panels to the fence posts using stainless steel 		5/16-inch x 4 ¾ inch mushroom head bolts, accompanied by security nuts and 		washers, with the help of a cover plate.

3.3	CLEANING

A.	Clean jobsite of excess materials.