SECTION 32 31 13

FENCE AND GATES

PART 1 – GENERAL

  - DESCRIPTION

- This section describes the following fence system:

- SecuriMesh D-Fence D-Lay 5+ M Vertical Internal Locking fencing panels fabricated with 	welded galvanized wire mesh including galvanized fence posts. SecuriMesh D-Fence D-	Lay 5+ M Vertical Internal Locking fence shall be furnished and installed as shown on the 	plans and specified herein, overall height of the fence shall be [ _ ] tall.

  - REQUIREMENTS

- Furnish materials, labor, expertise and equipment necessary to complete all work specified 	in this section and as shown on the drawings.

- Structural Performance: Provide product and installation capable of withstanding the 	effects of gravity and lateral loads and the following loads and stresses within limits and 	under conditions indicated.

    - Uniform pressure of 30LBS sq. ft. acting inward or outward.

    - Thermal Movements resulting from a temperature change (range) of 120 degrees Fahrenheit ambient and 180 degrees Fahrenheit material surfaces.

  - SUBMITTALS

- Shop drawings and manufacturer's literature: Provide specifications and construction detail 	drawings to substantiate quality of materials and provide details of fabrication and 	installation.

- Submittals shall be in accordance with standard construction practices to include complete 	detailed layout of all panels & posts.  Submittals shall include plan layout, elevations and 	section views of panels & posts.

- Certificate:  manufacturer's certification that materials meet specification requirements.

  - REFERENCES

- ANSI B36.10 & 36.19 – Standard Specification for the dimensions of the steel pipe.

- ASTM A123/A123M – Standard Specification for Steel, Zinc-Coated (Galvanized) by 	Hot-Dip Process

- ASTM D7091 –Standard Practice for Nondestructive Measurement of Dry Film Thickness 	of Nonmagnetic Coatings Applied to Ferrous Metals  and Nonmagnetic, Nonconductive 	Coatings Applied to Non-Ferrous Metals.

- ASTM D1729 –Standard Practice for Visual Appraisal of Colors and Color Differences of 	Diffusely-Illuminated Opaque Materials.

  - QUALITY ASSURANCE

- Installation of fence and materials shall conform to the requirements of the fence 	manufacturer.

- The fence shall be warranted from any defects in materials and workmanship for a period 	as specified in the relevant section of the contract documents.

PART 2 – PRODUCTS

  - MANUFACTURERS

- D-Fence D-Lay 5+ L Horizontal Internal Locking

Acceptable Manufacturers:

Bear Mountain Security, LLC

7400/7404 Gary Ave, Miami Beach, FL 33141

www.BearMountainSecurity.com

Tel: 202-549-3431

2.2       COMPONENTS

  - Panel:

    - Panel Height: [ _ ]

    - Panel Width: 10ft.

    - Panel shall be  of welded wire mesh of size 3 inch (horizontals) x 0.5 inch (verticals) with 	the wire of dia 11ga to 4ga.

  - Fence Posts:

    - Post shall be available in two types:

1.	H-Section comprising internal locking bolts, clips, clamps and angles to connect fence 	panels on the inside of the fence line.

2.	Proprietary SecuriFix post comprising internal locking bolts, clips, clamps and angles to 	connect fence panels on the inside of the fence line.

b.	The post shall be galvanized after fabrication. Top covered with pvc cap. Length as 	specified on the contract drawings.

c.	Post center spacing shall be 9.675ft.

  - Top Rail * Optional

    - Top rail shall be 2inch x 2inch x 0.196 inch equal angle.

  - Fittings and accessories:

    - The clamps & cover plates shall be anti-vandal and galvanized and sized as specified by 	the fence manufacturer.

PART 3 – EXECUTION

  - INSPECTION

  - Verify that final grading in fence location is completed and without irregularities which 	will interfere with fence installation. Security flat fence is designed to be installed on a 	level surface. Variations in height, slopes, stairs, and steeping shall be shown on contract 	drawings and on submittal drawings.

  - Field verify all fence dimensions and layout prior to commencing installation.

  - Do not commence work until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

- Install the fence following the manufacturer’s installation instructions; for detailed		guidance, please refer to the installation manual.

- Install fence plumb and level. Posts are to be embedded inside the ground.

- Do not install bent, bowed or otherwise damaged panels. Remove damaged	         	components from site and replace.

3.3	CLEANING

A.	Clean jobsite of excess materials.